# Topographic Radar Geometry-Guided Cross-Attention for Alpine Avalanche Debris Mapping in Sentinel-1 SAR: Cross-System Generalization Across the Alps, Pamirs, and Arctic

Abstract

Rapid, all-weather mapping of snow avalanche debris across rugged mountain terrain is critical for alpine civil protection, emergency response, and search-and-rescue operations. Synthetic Aperture Radar (SAR) sensors such as Sentinel-1 provide cloud-penetrating day-and-night winter monitoring, but bitemporal change detection in steep terrain suffers from foreshortening, layover, and radar shadow. We propose TopoRadar-Net, coupling bitemporal Sentinel-1 C-band SAR with local radar-topographic geometry through: (1) an analytical characterization of shift-invariant linear-convolution limits under non-stationary terrain projection; (2) multi-scale Radar-Topographic Cross-Attention Blocks; (3) a heuristic slope-prior loss audited for positive-label conflict; and (4) calibrated component- and corridor-level operational proxies. On AvalCD (7 events, 4 mountain systems, 993 polygons), TopoRadar-Net reaches 63.79%±1.88% pooled macro F1, compared with Attention U-Net (63.41%), SiamUNet-conc (63.13%), ResU-Net (62.92%), and SiamUNet-diff (54.87%). Across three seeds, only the margin over SiamUNet-diff is significant (+8.92%, p=0.021); the Attention U-Net difference is +0.38% (p=0.869). Seed-42 backslope differences do not persist as stable three-seed effects (Tromsø: −0.34%±6.28%; Pamir: +1.20%±7.73%). In Pamir, the false-alarm footprint is 30.0% lower than Attention U-Net (72.9 ha nominal-grid equivalent; 55.6 ha affine physical difference). The slope-prior mask overlaps 18.47% of training positives and lowers held-out positive recall inside that mask relative to No-GeoLoss. Validation-calibrated Pamir road-corridor alerts achieve 83.0% precision and 62.5% recall, but the single Tromsø road-intersecting avalanche is missed in all seeds. These results establish competitive pixel performance and explicit operational failure boundaries, not a component-isolated mechanism or deployment-ready system.

Keywords: Snow avalanche mapping; Sentinel-1 SAR; Mountain hazards; Radar-topographic geometry; Cross-attention network; Cross-system generalization; Disaster response

# Introduction

Snow avalanches represent one of the most destructive natural hazards in alpine and high-mountain environments worldwide, posing severe threats to human life, critical transportation corridors, hydropower infrastructure, and mountain settlements across major mountain systems such as the European Alps, the Pamirs, the Himalayas, and Arctic mountain belts (Bühler et al. 2019; Eckerstorfer et al. 2017; European Avalanche Warning Services 2022; Techel et al. 2018). Rapid, systematic, and regional-scale mapping of avalanche release zones and debris deposits is vital for emergency search-and-rescue operations, hazard index updating, and retrospective validation of numerical avalanche dynamics models (Tompkin and Leinss 2021; Eckerstorfer et al. 2021). However, conventional ground-based observations and optical satellite platforms (e.g., Sentinel-2, Landsat-8/9, PlanetScope) are frequently incapacitated during major avalanche cycles due to persistent cloud cover, winter blizzards, and polar night in high-latitude mountain regimes (Eckerstorfer et al. 2019; Vickers et al. 2016).

Synthetic Aperture Radar (SAR), particularly spaceborne C-band radar operating on the European Space Agency’s (ESA) Sentinel-1 constellation, provides an indispensable all-weather, day-and-night observation system with a regular 6- to 12-day repeat cycle (Eckerstorfer et al. 2017; Small 2011). When snow avalanches release, dense slab flow and turbulent powder snow transport heavy snow masses down mountain chutes, depositing rough, granulated debris mounds in runout fans. This mechanical deposition alters the snowpack surface from smooth, undisturbed snow into rough, blocky debris, markedly increasing surface and volume backscatter (Δσ0=σpost0−σpre0>0) in both cross-polarization (VH) and co-polarization (VV) channels (Tompkin and Leinss 2021).

Despite these promising physical principles, automated SAR change detection in rugged alpine terrain has historically suffered from acute false alarm rates and severe geometric distortions (Bianchi et al. 2021; Gatti et al. 2026). In steep topography, the side-looking imaging geometry of SAR creates complex radiometric distortions:

- Local Incidence Angle (LIA) Compression: Mountain slopes tilted directly toward the radar antenna experience extreme ground-range compression (foreshortening) and layover, resulting in artificially high backscatter values that mimic avalanche debris. Conversely, slopes tilted away from the sensor produce high incidence angles and radar shadow, where signal-to-noise ratios collapse (Small 2011; Tompkin and Leinss 2021).

- Directional Slope-Aspect Alignment: Sentinel-1 operates in near-polar orbits with fixed ascending (look azimuth ϕlook≈78∘) and descending (ϕlook≈282∘) lines of sight. As established by Tompkin and Leinss (2021), avalanche backscatter contrast is highly anisotropic with respect to the angle between slope aspect and radar look direction: slopes facing away from the radar exhibit the highest relative brightness anomalies (55∘±20∘ LIA), while foreslopes generate severe topographic clutter. In complex alpine relief, steep topography also influences winter terrain roughness and snow drift accumulation (Veitinger et al. 2014).

- Slope-Prior Sensitivity: Flat valley bottoms (<5∘) and very steep rock faces (>65∘) can contain clutter, but they also contain genuine mapped debris through runout, terrain-model error, or geolocation effects. Any slope prior must therefore be treated as a heuristic rather than a physical-validity mask.

Existing automated detection approaches have struggled to address these coupled physical mechanisms. Early methods relied on empirical thresholding of backscatter differences (Δσ0) combined with heuristic slope filters (Eckerstorfer et al. 2019; Vickers et al. 2016); Liu et al. (2021) subsequently combined object-based Sentinel-1 classification with topographic causative indices using SVM and logistic-regression mapping. Recent deep-learning work introduced Siamese convolutional networks, attention-augmented change detection (Caye Daudt et al. 2018; Zhang et al. 2018; Chen et al. 2022; Bandara and Patel 2022; Gatti et al. 2026), and promptable foundation models (Gelato et al. 2026). Bianchi et al. (2021) investigated convolutional segmentation with slope and potential-angle-of-reach masks, while Gatti et al. (2026) explored multimodal Swin Transformer concatenation on AvalCD. TopoRadar-Net differs from the object-based SVM/LR workflow of Liu et al. (2021) through learned SAR-query/topography-key-value fusion and differs from single-configuration studies through cross-system, multi-seed stability and negative-result characterization. In the wider natural-hazards literature, geomorphological assessment and deep-learning susceptibility studies show severe transfer hurdles across catchments (Wang et al. 2023; Sajjad et al. 2024; Zhou et al. 2023; Guo et al. 2025; Guo et al. 2024; Garova et al. 2025; Cao et al. 2026). Our controlled comparisons evaluate whether auxiliary radar-topographic cross-attention and continuous aspect-look coordinates provide measurable advantages over standard architectures.

To overcome these fundamental limitations, we propose TopoRadar-Net, a lightweight, physics-guided cross-attention architecture specifically engineered for alpine SAR change detection. Rather than relying on black-box concatenation, TopoRadar-Net introduces four methodological, theoretical, and operational innovations:

- Theoretical Formulation of 2D Convolutional Inadequacy: We formulate the geometric limitations of shift-invariant 2D linear convolutions operating purely on SAR intensity space under the non-stationary radiometric projection field cosψsinθinc. We show that projecting terrain into a continuous 4D aspect-look manifold M provides continuous orientation coordinates to condition feature representations without circular angular branch-cut discontinuities.

- Multi-Scale Radar-Topographic Cross-Attention Block (RTCAB): We design a cross-attention mechanism wherein multi-scale bitemporal SAR change features serve as queries to interrogate regionally-pooled topographic geometry keys and values, conditioning spatial feature representations on terrain context.

- Heuristic Geomorphic Slope Prior (Geo-Loss): We evaluate a soft penalty on predictions below 5∘ and above 65∘. A construct audit shows that the mask also covers genuine positive labels, so its ablation is interpreted as component sensitivity and its subgroup trade-off is reported explicitly.

- Cross-System Generalization and Conceptual Decision Support: We evaluate TopoRadar-Net across four mountain systems comprising 7 Sentinel-1 scenes and 993 avalanche polygons. Across three seeds, the pooled margin over Attention U-Net is +0.38% (p=0.869), while the margin over SiamUNet-diff is significant (+8.92%, p=0.021). Exploratory Seed-42 backslope differences do not persist as stable three-seed effects (Tromsø: −0.34%±6.28%; Pamir: +1.20%±7.73%), delimiting the evidence for directional conditioning. An explicitly unvalidated triage framework accompanies a 30.0% Pamir false-alarm difference (72.9 ha nominal-grid equivalent; 55.6 ha affine physical difference) and 1.80 s model-forward latency.

The remainder of this article is organized as follows: Section 2 describes the regional settings, the AvalCD benchmark dataset, and data preprocessing. Section 3 details the radar-topographic physics, the TopoRadar-Net architecture, and the loss formulation. Section 4 presents comparative baseline performance, cross-region zero-shot evaluations, instance-level EAWS detection, and ablation studies. Section 5 discusses physical mechanisms, operational mountain safety implications, and study limitations. Section 6 concludes the paper.

# Study Areas and Dataset

## Geographic and Climatic Characteristics of the Four Mountain Systems

To rigorously assess the generalizability of deep change detection models across diverse mountain environments, this study leverages the open-access AvalCD benchmark (Gatti 2026; Gatti et al. 2026). The benchmark spans four prominent, geographically disparate mountain ranges across Europe, Central Asia, and the Arctic (Table 1):

Table 1. Summary of the AvalCD benchmark across four global mountain systems and seven avalanche events.

| Mountain System | Acquisition Date | Region / Country | Elevation (m) | Grid Size (px) | Polygons | Debris (px) |
| --- | --- | --- | --- | --- | --- | --- |
| European Alps | 3 Apr 2024 | Lombardy, Italy | 1,250–3,890 | 2462 × 2091 | 231 | 32,606 |
|  | 29 Jan 2025 | Lombardy, Italy | 1,310–3,890 | 2388 × 1629 | 207 | 11,438 |
|  | 18 Mar 2025 | Lombardy, Italy | 1,210–3,890 | 3069 × 2814 | 79 | 7,392 |
| Greenland Arctic | 13 Apr 2016 | Southwest Greenland | 0–1,620 | 2387 × 4619 | 117 | 64,822 |
|  | 11 Apr 2021 | Southwest Greenland | 0–1,740 | 2836 × 4931 | 129 | 81,595 |
| Pamir Mountains | 21 Feb 2023 | Shughnon, Tajikistan | 2,100–5,420 | 1894 × 1896 | 113 | 22,579 |
| Scandinavian Arctic | 20 Dec 2024 | Troms, Norway | 0–1,830 | 1691 × 1454 | 117 | 33,972 |
| Total / Benchmark | 7 Events | 4 Ranges | 0–5,420 | 48.7M px | 993 | 254,404 |

- European Alps (Livigno, Italy): Located in the central Italian Alps near the Swiss border (46∘32′N, 10∘08′E), the Livigno region represents a temperate alpine mountain system characterized by steep glacial troughs, U-shaped valleys, and elevations ranging from 1,250 m to over 3,890 m. The region features intensive human recreation, ski infrastructure, and transportation corridors. The dataset encompasses three distinct avalanche cycles: a spring slab cycle in April 2024, a mid-winter powder cycle in January 2025, and a complex wet-snow avalanche cycle in March 2025.

- Pamir Mountains (Pish, Tajikistan): The Pamir knot in Central Asia (37∘35′N, 71∘39′E) represents an extreme high-altitude, continental mountain regime characterized by deep arid valleys, narrow gorges, and rugged peaks exceeding 5,400 m in relief. In February 2023, extreme anomalous snowfall triggered a catastrophic avalanche cycle across the Shughnon District, resulting in severe infrastructure destruction and humanitarian emergencies. The snowpack is continental and facetted, with avalanche paths spanning massive vertical descents (>2,000 m).

- Scandinavian Arctic (Tromsø, Norway): Situated at 69∘39′N, 18∘57′E in Northern Norway, this region features coastal maritime Arctic mountains rising directly from fjords to rugged alpine massifs. The snowpack is heavily influenced by cyclonic storms, freezing rain, and high winds, producing complex slab avalanches and wet debris. The Tromsø inventory provides detailed polygon annotations categorized according to the European Avalanche Warning Services (EAWS) destructive size scale (classes D1 through D4).

- Greenland Sub-Polar Alpine (Nuuk, Greenland): Surrounding the capital region of Nuuk (64∘11′N, 51∘44′W), this coastal sub-polar environment consists of rugged coastal mountains, deep fjords, and permafrost terrain. Two events (April 2016 and April 2021) capture widespread avalanche releases in a polar maritime regime under intense wind-drift conditions.

## Remote Sensing and Topographic Data Modalities

For each of the seven events, the benchmark provides co-registered, multi-modal geospatial rasters:

- Sentinel-1 C-band SAR Imagery: Dual-polarization intensity data (co-polarization VV and cross-polarization VH) acquired in Interferometric Wide (IW) swath mode. For each event, a pre-event reference acquisition and a post-event acquisition are preprocessed via the ESA SNAP toolbox, applying orbit state vector updates, border noise removal, calibration to backscatter coefficient σ0 (in decibels, dB), and Range-Doppler terrain correction at 10 m spatial resolution.

- Local Incidence Angle (LIA): Exact per-pixel local incidence angle rasters computed from satellite orbit geometry and the digital elevation model, quantifying the angle between the surface normal and the incoming radar wave.

- Copernicus DEM (GLO-30): High-quality 30 m global digital elevation model resampled to the 10 m Sentinel-1 SAR grid, providing continuous elevation measurements across all alpine catchments.

- Terrain Slope and Aspect: Derived terrain slope (θslope∈[0∘,90∘]) and aspect (α∈[0∘,360∘]) rasters extracted using 2D spatial gradients from the Copernicus DEM.

- Ground Truth Validation Inventories: Manual expert delineations combining optical satellite confirmation, field reports, and local avalanche bulletin records (Gatti 2026), delivered both as raster masks ({0,1}) and vector GeoPackages containing exact polygon boundaries and metadata.

## Partitioning for Cross-System Generalization

To limit spatial autocorrelation and characterize cross-system transfer, we partition the dataset at the regional mountain-system level. The European Alps (Livigno 2024 and Jan 2025) and Greenland (Nuuk 2016) form the training cohort (555 polygons). Model selection and threshold calibration use only Livigno March 2025 and Nuuk 2021 (208 polygons). Tromsø (117 polygons) and Pamir (113 polygons) are held out from training and selection. Table 8 uses the finite-raster valid masks: 2,311,510 pixels in Tromsø and 3,503,382 in Pamir, equivalent to 231.151 and 350.338 km2 on the nominal 10 m grid. At the local affine pixel areas (85.15 and 76.34 m2/px), the valid footprints are 196.820 and 267.435 km2. Numerator and denominator scale together, so TopoRadar-Net’s false-alarm densities are 0.38 and 0.48 ha/km2 under either area convention.

# Methodology

## Physical Principles of Radar-Topographic Interaction

In spaceborne Synthetic Aperture Radar (SAR), the calibrated radar backscatter coefficient σ0 reflects the complex interaction between transmitted microwave pulses (wavelength λ≈5.55 cm for Sentinel-1 C-band) and surface dielectric and geometric roughness properties (Small 2011). When a slab avalanche releases, the rapid down-slope displacement of snow breaks the cohesive winter mantle into fragmented clods and chaotic debris piles. In dry snow regimes, smooth undisturbed snow behaves largely as a volume-scattering or specular surface, yielding low backscatter (σVH0≈−20 dB to −24 dB) (Tompkin and Leinss 2021), while terrain topography influences winter snow depth and microtopographic roughness (Veitinger et al. 2014). In contrast, granulated avalanche debris exhibits intense diffuse surface scattering and multiple internal reflections, typically resulting in an anomalous positive backscatter delta:

ΔσVH0=σpost,VH0−σpre,VH0ΔσVV0=σpost,VV0−σpre,VV0

Additionally, depolarizing volume scattering in rough debris modifies the cross-polarization ratio, yielding an informative differential metric:

ΔCR=(σpost,VH0−σpost,VV0)−(σpre,VH0−σpre,VV0)

However, in steep alpine topography, this backscatter anomaly is heavily modulated by local radar illumination geometry (Tompkin and Leinss 2021). Let n=(−sinθsinα,−sinθcosα,cosθ)T denote the terrain surface unit normal defined by slope steepness θ and aspect azimuth α. Let s=(sinθincsinϕlook,sinθinccosϕlook,cosθinc)T denote the incident radar unit vector pointing toward the satellite antenna at nominal incidence angle θinc and look azimuth ϕlook. The cosine of the local incidence angle (LIA) ψ is governed by their inner product:

cosψ=n⋅s=cosθcosθinc−sinθsinθinccos(α−ϕlook)  (4)

Eq. (4) reveals that local incidence angle is intrinsically coupled to the directional alignment term θalign=cos(α−ϕlook). When θalign>0, the slope faces toward the sensor (foreslope), driving ψ→0∘ and causing intense radiometric compression (foreshortening/layover) that generates spurious high-backscatter ridges. Conversely, when θalign<0, the slope faces away from the sensor (backslope), producing large incidence angles (ψ>50∘) where true avalanche debris exhibits the highest relative contrast against the dark background snowpack (Tompkin and Leinss 2021). If ψ≥90∘, true radar shadow occurs.

To provide the neural network with a complete, continuous, and singularity-free representation of terrain geometry without angular modular discontinuities at 0∘/360∘, we define the continuous topographic coordinate vector g(u,v)∈R6 at each spatial location (u,v):

g(u,v)=[cosψ, θθref, sinα, cosα,H−μHσH, cos(α−ϕlook)]T  (5)

where θref=60∘ is a characteristic alpine scaling factor, and μH,σH normalize digital elevation H.

## Physical and Geometric Formulation of Radar-Topographic Distortion

To formalize why standard 2D convolutional and self-attention backbones face inherent geometric limits in rugged alpine terrain, we analyze the mapping between terrain orientation and observed backscatter intensity.

Proposition 1 (Geometric Radar-Topographic Illumination Modulation). Let z=H(u,v) be a topographic surface defined on spatial coordinates (u,v)∈Ω⊂R2, illuminated by an incident radar look vector s(θinc,ϕlook). Under radiometric terrain modeling (Small 2011), observed radar backscatter across sloped terrain is influenced by local ground scattering area. Under first-order facet geometry, the ratio between nominal flat-surface ground area and local tilted facet area is conventionally characterized by the geometric modulation factor:

G(u,v)≈cosψ(u,v)sinθinc

where the local incidence angle ψ(u,v) satisfies Eq. (4), with local slope θ=arctan(∥∇H(u,v)∥) and aspect α=atan2(−∂uH,−∂vH).

Proposition 2 (Geometric Ambiguity of Shift-Invariant 2D Linear Convolutions). Let K*I denote any 2D spatial convolution with shift-invariant kernel K operating purely on bitemporal SAR channels I=(σpre0,σpost0). Because the geometric modulation factor G(u,v) depends on localized directional aspect alignment cos(α(u,v)−ϕlook), it is spatially non-stationary and lacks translational symmetry across arbitrary mountain ridgelines. On steep radar-facing foreslopes (θalign>0,ψ→0∘), foreshortening compression elevates backscatter, producing bright spatial textures that resemble rough avalanche debris. Conversely, on backslopes facing away from the radar (θalign<0,ψ>50∘), the background snowpack is radiometrically dark. Consequently, a spatially shift-invariant 2D linear convolutional kernel operating exclusively on SAR intensity channels lacks the orientation-conditioned parameters necessary to dynamically normalize G(u,v) across opposing mountain facets.

Proposition 3 (Continuous Aspect-Look Representation via Manifold M). Representing directional terrain coordinates via the continuous vector M(u,v)=[sinα,cosα,cosψ,θalign]T∈[−1,1]4 removes the 0∘/360∘ circular branch-cut discontinuity of modular aspect representation α (mod 360∘). In TopoRadar-Net, the multi-scale cross-attention mechanism uses SAR change features to query these regionally pooled geometric representations, dynamically conditioning feature activations on local terrain orientation.

## TopoRadar-Net Architecture

Fig. 1. Overall architecture of TopoRadar-Net. Pre- and post-event Sentinel-1 SAR intensity pairs are processed by a multi-scale Siamese residual encoder. Concurrently, a topographic feature pyramid extracts hierarchical representations from the 6-channel radar-geometry vector. At each decoder stage, the Radar-Topographic Cross-Attention Block (RTCAB) uses SAR difference features to query regionally-pooled terrain geometry, followed by spatial gating that conditions pixel features on terrain context. This simplified training schematic omits the ground-truth target input to Geo-Loss for clarity.

Fig. 1 illustrates the overall architecture of TopoRadar-Net. The network consists of three tightly integrated components: (1) a Siamese multi-scale SAR feature encoder; (2) a multi-scale topographic feature pyramid; and (3) hierarchical Radar-Topographic Cross-Attention Blocks (RTCAB) embedded within the progressive upsampling decoder.

### Siamese Multi-Scale SAR Feature Encoder

The pre-event SAR image Xpre∈R2×H×W and post-event SAR image Xpost∈R2×H×W (containing dual-polarization VH and VV channels) are passed through four hierarchical residual convolutional stages with shared weights. Each stage l∈{1,2,3,4} comprises two 3×3 convolutional layers with Batch Normalization, GELU activations, and a residual skip connection:

Fprel=El(Fprel−1), Fpostl=El(Fpostl−1)

yielding feature representations at spatial resolutions H/2l−1×W/2l−1 with channel dimensions Cl∈{32,64,128,256}.

At each scale, the bi-temporal features are fused by concatenating the signed difference, the post-event features, and the pre-event features, followed by a 1×1 linear projection:

Fdiffl=Conv1×1Fpostl−Fprel, Fpostl, Fprel

This formulation preserves both directional backscatter changes (Fpostl−Fprel) and baseline radiometric context (Fprel,Fpostl).

### Topographic Feature Pyramid

In parallel, the continuous 6-channel terrain geometry tensor G∈R6×H×W defined in Eq. (5) is processed by a dedicated lightweight multi-scale convolutional subnetwork:

Tl=Tl(Tl−1), l∈{1,2,3,4}

producing hierarchical topographic feature maps Tl∈RCl×Hl×Wl matching the spatial resolution and channel depth of the SAR difference features.

### Radar-Topographic Cross-Attention Block (RTCAB)

To condition the SAR change representations on terrain geometry without memory explosion, we design the RTCAB module. Rather than computing full N×N self-attention (which scales quadratically with Hl×Wl and exceeds GPU memory on high-resolution satellite tiles), RTCAB treats the full-resolution SAR change features as Queries (Q), while adaptively pooling the topographic geometry into a regional spatial context grid of size M=k×k (where k=8):

Q=FdifflWQ∈RB×N×Cl (N=Hl×Wl)K=Poolk×k(Tl)WK∈RB×M×Cl (M=k2)V=Poolk×k(Tl)WV∈RB×M×Cl

Cross-attention weights A∈RB×N×M are computed across multiple attention heads (h∈{1,2,4}):

A=softmaxQKTdk

The attended geometric context O=AV allows every SAR pixel to interrogate whether surrounding regional slope and aspect angles physically support avalanche accumulation.

To further inject pixel-level boundary constraints, we compute a dynamic spatial topographic gate:

Wgate=σConv1×1GELUConv3×3(Tl)

The final conditioned feature map is computed as:

Fl=BatchNormFdiffl+O⊙(1+Wgate)

This dual mechanism allows broad macro-topographic routing via cross-attention alongside sharp per-pixel edge modulation via spatial gating.

## Heuristic Geomorphic Slope-Prior Loss

Standard semantic segmentation losses do not encode a slope-dependent prior. We therefore tested a heuristic regularizer that discourages probability mass below 5∘ and above 65∘, terrain ranges expected to contain fewer debris pixels on average. These ranges are not physically impossible: runout deposits and mapping/geolocation effects place genuine positive labels inside the penalty mask.

The resulting Geomorphically Bounded Loss (Geo-Loss) is:

L=LBCE+λdiceLDice+λgeomLgeom

where LBCE is positive-weighted binary cross-entropy (with positive weight wpos=3.0 to combat the extreme class imbalance of sparse avalanche debris):

LBCE=−1|Ω|i∈Ω​wposyilogyi+(1−yi)log(1−yi)

and LDice is the soft Dice loss:

LDice=1−2i∈Ω​yiyi+ϵi∈Ω​yi+i∈Ω​yi+ϵ

The geomorphic penalty term Lgeom is defined as:

Lgeom=i∈Ω​yi⋅I(θi<5∘∨θi>65∘)i∈Ω​I(θi<5∘∨θi>65∘)+ϵ

where yi=σ(zi) is the predicted probability, θi is terrain slope in degrees, and I(⋅) is an indicator function. With λgeom=0.5, predictions inside the slope-prior mask receive a direct penalty regardless of the target label. BCE and Dice therefore compete with this term on positive pixels inside the mask; the construct is evaluated empirically as a heuristic prior rather than a physical-validity constraint.

## Training Protocol and Optimization Details

All models are implemented in PyTorch and trained on an Apple Silicon unified memory GPU environment using Metal Performance Shaders (MPS). Patch-based training utilizes 128×128 pixel spatial windows extracted with a 64-pixel stride. Training patches are balanced with a 1:1 ratio between positive patches (containing ≥5 avalanche debris pixels) and negative background patches, yielding 2,054 training patches per epoch across the three training events (Livigno 2024, Livigno Jan 2025, Nuuk 2016). Sentinel-1 ascending acquisitions feature a canonical nominal look azimuth of ϕlook≈78∘, which defines the aspect-look alignment coordinate θalign=cos(α−78∘). The executed checkpoint-era augmentation applies random horizontal reflections (negating aspect sine), vertical reflections (negating aspect cosine), and random orthogonal rotations to all spatial arrays; the precomputed orbital look-alignment channel is spatially transformed but not directionally recomputed. This convention exactly identifies the released training path, but it is not a fully rotation/reflection-equivariant transformation of the directional manifold. We therefore interpret the directional-channel findings as empirical sensitivity results rather than proof of an equivariant physical mechanism.

To evaluate the specific architectural benefits of multi-scale radar-topographic cross-attention and continuous aspect-look manifold modeling under controlled conditions, all comparative baselines and TopoRadar-Net variants are trained using the identical optimization schedule and loss formulation (Eq. 18), with λgeom=0.5 (except in the No-GeoLoss ablation where λgeom=0). Network parameters are optimized using AdamW (β1=0.9,β2=0.999, weight decay 10−4) with an initial learning rate of 2×10−4 decayed via cosine annealing over 6 epochs (batch size 16) with gradient clipping at norm 5.0. Checkpoint selection is governed by balanced patch-level validation F1-score evaluated at threshold 0.5 on the validation cohort (Livigno March 2025 and Nuuk 2021). Subsequently, the operating decision threshold τ is calibrated on full-scene validation rasters using a grid search over τ∈[0.20,0.80] in steps of 0.02, maximizing validation F1. This frozen threshold is then applied directly to full-scene zero-shot inference on Tromsø and the Pamir Mountains using sliding-window inference with 2D Hanning-window blending (stride 64). Full-scene sliding-window inference over a 2,500×2,000 pixel alpine scene (1,200 patches at stride 64) requires approximately 1.80±0.02 seconds on Apple MPS hardware, with TopoRadar-Net maintaining a compact parameter footprint of 3.46M parameters (3,455,729 parameters).

After the primary evaluation, we conducted a labelled post-hoc operational proxy. Predicted components of at least 10 pixels were assigned mean scores; a one-feature Platt model and an F2-optimal alert threshold were fit only on the two validation scenes. Frozen alerts on test scenes were intersected with 30 m buffers around a committed OpenStreetMap major-road snapshot and counted correct at ≥30% ground-truth component overlap. Cached end-to-end timing includes raster loading, road rasterization, inference, calibration, component extraction, and intersection, but excludes data acquisition and human review. This exploratory protocol measures calibration and corridor proximity, not dispatch or stakeholder benefit.

# Results

## Cross-System Performance on Unseen Mountain Systems

To characterize cross-system transfer, all models are trained on European Alps (Livigno) and Greenland (Nuuk) events and evaluated without site-specific tuning on two held-out scenes: Tromsø, Norway, and Pish, Tajikistan. Table 2 reports pixel-level metrics across three independent training seeds (42, 123, and 456; mean ± standard deviation).

Table 2. Cross-system zero-shot evaluation on unseen mountain systems (Scandinavian Arctic Tromsø and Central Asian Pamir Mountains), reporting Mean ± Standard Deviation across 3 independent training seeds. All values recomputed directly from primary experiment logs and serialized model weights.

| Method | Params | Scandinavian Arctic (Tromsø) |  |  | Pamir Mountains (Pish) |  | Pooled |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | F1 (%) | IoU (%) | HR@0.3 | F1 (%) | IoU (%) | Macro F1 (%) |
| SiamUNet-diff | 2.01M | 72.64 ± 0.39 | 57.03 ± 0.48 | 63.25 ± 2.52 | 37.11 ± 2.44 | 22.81 ± 1.86 | 54.87 ± 1.03 |
| SiamUNet-conc | 2.36M | 78.92 ± 1.07 | 65.20 ± 1.46 | 82.62 ± 1.07 | 47.34 ± 0.70 | 31.02 ± 0.60 | 63.13 ± 0.79 |
| ResU-Net | 2.01M | 76.60 ± 1.30 | 62.09 ± 1.71 | 76.92 ± 4.83 | 49.25 ± 3.39 | 32.74 ± 2.96 | 62.92 ± 1.42 |
| Attention U-Net | 2.29M | 78.09 ± 0.99 | 64.07 ± 1.33 | 79.20 ± 2.45 | 48.73 ± 2.74 | 32.26 ± 2.38 | 63.41 ± 1.03 |
| TopoRadar-Net (Ours, S42) | 3.46M | 79.35 | 65.76 | 74.36 | 51.23 | 34.44 | 65.29 |
| TopoRadar-Net (Ours, Mean) | 3.46M | 77.40 ± 1.86 | 63.16 ± 2.45 | 75.50 ± 1.61 | 50.18 ± 4.40 | 33.61 ± 3.89 | 63.79 ± 1.88 |

Table 2 shows region- and seed-dependent transfer. In Pamir, TopoRadar-Net records 50.18%±4.40% F1, compared with Attention U-Net (48.73%±2.74%), ResU-Net (49.25%±3.39%), SiamUNet-conc (47.34%±0.70%), and SiamUNet-diff (37.11%±2.44%). On the pooled two-system benchmark it records 63.79%±1.88%, compared with 63.41%±1.03%, 63.13%±0.79%, 62.92%±1.42%, and 54.87%±1.03%, respectively. Across seeds, only the difference from SiamUNet-diff is significant; comparisons with the three competitive baselines are not.

On the Scandinavian Arctic test scene (Tromsø), TopoRadar-Net reaches 77.40%±1.86% pixel F1 and 63.16%±2.45% IoU, compared with ResU-Net (76.60%±1.30%), SiamUNet-diff (72.64%±0.39%), and SiamUNet-conc (78.92%±1.07%). SiamUNet-conc then records 47.34% in Pamir, a region-dependent drop compatible with sensitivity to the changed mountain regime; the two-region contrast does not identify overfitting to a single factor.

Fig. 2. Quantitative cross-system generalization performance across evaluated deep architectures on unseen mountain ranges (Scandinavian Arctic in Tromsø and Central Asian Pamir Mountains in Tajikistan), reporting Mean ± Standard Deviation across 3 independent training seeds.

## Statistical Significance Testing

Because satellite remote sensing pixels within mountain regions exhibit spatial autocorrelation, naive pixel-level hypothesis tests artificially underestimate uncertainty. To evaluate spatial sampling variability, we execute a 1,000-draw paired cluster-bootstrap test over 128×128 spatial blocks of the held-out test scenes. While adjacent raster tiles may retain residual boundary spatial dependence, block bootstrapping provides a substantially more realistic assessment of spatial-sampling variance than unclustered pixel resampling. Table 3 summarizes the paired bootstrap estimates, 95% confidence intervals, and empirical two-tailed p-values.

Table 3. Paired statistical significance testing of TopoRadar-Net versus baselines: (a) 1,000-draw spatial block cluster bootstrap on the held-out Scandinavian Arctic test scene (Seed 42, n=143 blocks of size 128 × 128); (b) 1,000-draw spatial block cluster bootstrap pooling all 339 geographic blocks across both unseen mountain regimes (Seed 42, pixel-pooled micro-F1); and (c) Paired t-test across 3 independent training seeds on pooled macro-F1.

| Comparison | Mean ΔF1 (%) | 95% CI Lower | 95% CI Upper | p-value |
| --- | --- | --- | --- | --- |
| Part A: Single-Scene Regional Significance (Spatial Block Bootstrap, Tromsø, Seed 42) |  |  |  |  |
| vs. SiamUNet-diff | +6.39% | +2.59% | +10.59% | <0.001 |
| vs. ResU-Net | +4.35% | +1.33% | +7.38% | 0.010 |
| vs. Attention U-Net | +2.30% | +0.27% | +4.41% | 0.030 |
| vs. SiamUNet-conc | -1.12% | -2.80% | +0.40% | 0.144 |
| vs. Ablation (No-LIA) | +2.04% | +0.98% | +3.13% | <0.001 |
| vs. Ablation (No-CrossAttn) | +1.42% | +0.33% | +2.60% | 0.010 |
| vs. Ablation (No-GeoLoss) | +3.16% | +1.49% | +4.75% | 0.002 |
| vs. Ablation (No-Aspect) | -0.09% | -1.39% | +1.04% | 0.942 |
| Part B: Multi-Region Spatial Sampling (339 Blocks, Seed 42, Micro-F1) |  |  |  |  |
| vs. SiamUNet-diff | +12.77% | +9.18% | +16.61% | <0.001 |
| vs. ResU-Net | +4.30% | +1.54% | +6.97% | 0.002 |
| vs. Attention U-Net | +3.27% | +1.12% | +5.37% | 0.010 |
| vs. SiamUNet-conc | +2.47% | +0.41% | +4.60% | 0.026 |
| Part C: Multi-Seed Training Uncertainty (Paired t-Test across 3 Seeds, Macro-F1) |  |  |  |  |
| vs. SiamUNet-diff | +8.92% | +3.25% | +14.59% | 0.021 |
| vs. ResU-Net | +0.86% | -3.23% | +4.96% | 0.459 |
| vs. SiamUNet-conc | +0.65% | -4.69% | +5.99% | 0.651 |
| vs. Attention U-Net | +0.38% | -8.28% | +9.04% | 0.869 |

The bootstrap confidence intervals and seed-level tests distinguish spatial-sampling stability from training-seed variance. In the coastal Scandinavian Arctic setting (Part A), against Attention U-Net, TopoRadar-Net records a mean paired regional difference of +2.30% with a 95% CI of [+0.27%,+4.41%] (p=0.030), +4.35% versus ResU-Net (p=0.010), and +6.39% versus SiamUNet-diff (p<0.001). Under spatial block resampling across all 339 spatial blocks conditional on Seed 42 (Part B), the micro-F1 differences are Attention U-Net +3.27%, ResU-Net +4.30%, SiamUNet-conc +2.47%, and SiamUNet-diff +12.77%.

Repeating the pooled 339-block comparison with an independently seeded bootstrap for every training seed gives +3.26% (95% CI [+1.32,+5.33], p=0.002), −2.99% ([−6.67,+0.08], p=0.060), and +4.16% ([+1.00,+7.27], p=0.016) for Seeds 42, 123, and 456, respectively. The Seed-42 value differs from Part B’s +3.27% only because the primary and seed-repeated analyses use independent deterministic bootstrap streams over the same 339 blocks. The descriptive across-seed mean is +1.48%±3.18%, positive in two of three seeds. Thus the Seed-42 spatial interval is not a substitute for training-seed uncertainty.

The paired t-test across three seeds over pooled macro-F1 (Part C) likewise finds no significant difference versus Attention U-Net (+0.38%, p=0.869), ResU-Net (+0.86%, p=0.459), or SiamUNet-conc (+0.65%, p=0.651); only SiamUNet-diff differs significantly (+8.92%, 95% CI: [+3.25%,+14.59%], p=0.021). Attention U-Net leads on Seed 123 (64.78% vs 61.14%), showing that initialization changes the competitive ordering. Part-A component comparisons quantify Seed-42 sensitivity to No-LIA (+2.04%, p<0.001), No-CrossAttn (+1.42%, p=0.010), and No-GeoLoss (+3.16%, p=0.002), without isolating causal mechanisms.

## Instance-Level Detection Completeness across EAWS Avalanche Sizes

For alpine risk assessment and emergency management, detecting individual avalanche release and runout polygons is critical (European Avalanche Warning Services 2022; Techel et al. 2018). Table 4 evaluates instance-level detection completeness on the Tromsø inventory (117 polygons), categorized by European Avalanche Warning Services (EAWS) destructive size classes: Class D1 (Small, n=5), Class D2 (Medium, n=25), Class D3 (Large, n=71), and Class D4 (Very Large, n=16).

Table 4. Instance-level avalanche detection hit rates across EAWS size classes on the Tromsø benchmark (n=117 polygons), evaluated at minimum 30% (HR@0.3) and 50% (HR@0.5) polygon area overlap across true 3-seed means.

| Method | Overall Hit Rate |  | Size-Disaggregated Hit Rate (HR@0.3) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | HR@0.3 (%) | HR@0.5 (%) | D1 (n=5) | D2 (n=25) | D3 (n=71) | D4 (n=16) |
| SiamUNet-diff | 63.25 ± 2.52% | 54.42 ± 2.80% | 6.7% | 33.3% | 71.8% | 89.6% |
| SiamUNet-conc | 82.62 ± 1.07% | 74.07 ± 1.40% | 13.3% | 61.3% | 91.1% | 100.0% |
| ResU-Net | 76.92 ± 4.83% | 68.09 ± 4.20% | 6.7% | 53.3% | 85.9% | 95.8% |
| Attention U-Net | 79.20 ± 2.45% | 69.52 ± 2.10% | 6.7% | 58.7% | 86.9% | 100.0% |
| TopoRadar-Net (Ours) | 75.50 ± 1.61% | 68.09 ± 1.95% | 20.0% | 49.3% | 84.0% | 95.8% |

At the operational threshold (≥30% polygon overlap), TopoRadar-Net achieves an overall hit rate of 75.50%±1.61% on the held-out Scandinavian Arctic test scene (reaching 87, 87, and 91 detected polygons across the three seeds; 3-seed mean 88.3/117), with 95.8% detection completeness on the largest Class D4 avalanches (100% on seeds 42 and 123; 87.5% on seed 456; 3-seed mean 95.8%), 84.0% completeness on Class D3 avalanches, 49.3% on Class D2, and 20.0% on Class D1. On the extreme high-relief Pamir Mountains (Pish, Tajikistan), polygon-level metadata lacks EAWS size classifications, resulting in zero matching polygon boundaries under EAWS metrics (0.0%), whereas pixel-level F1 is preserved at 50.18%±4.40% (reaching 54.97%), maintaining physical sensitivity across massive 2,000 m vertical chutes.

Fig. 3. Instance-level avalanche detection completeness across European Avalanche Warning Services (EAWS) destructive size classes (D1 through D4) on the held-out Scandinavian Arctic test benchmark (n=117 polygons), evaluated at minimum 30% polygon overlap (HR@0.3).

## Systematic Architectural Ablation Study

To evaluate sensitivity to each input and architectural component, we conduct an ablation study across four controlled variants, each trained and evaluated under the identical multi-seed protocol across both unseen test regions and the pooled benchmark (Table 5):

- Ablation 1 (No-LIA): Local incidence angle conditioning is omitted (cosψ=0), forcing the model to rely only on terrain slope and aspect.

- Ablation 2 (No-Aspect): The 2D aspect vector [sinα,cosα] and radar look alignment (θalign) are zeroed out, depriving the network of directional look orientation.

- Ablation 3 (No-CrossAttn): The multi-scale cross-attention mechanism is replaced with simple channel concatenation of topographic features followed by 1×1 convolutions, exactly mimicking naive multimodal stacking.

- Ablation 4 (No-GeoLoss): The geomorphically bounded loss is removed (λgeom=0), training exclusively on standard weighted BCE and Dice losses.

Table 5. Systematic ablation analysis of TopoRadar-Net components across unseen mountain test regions and the pooled benchmark (Mean ± Standard Deviation across 3 independent training seeds).

| Configuration | Scandinavian Arctic (Tromsø) |  |  | Pamir Mountains (Pish) |  | Pooled |
| --- | --- | --- | --- | --- | --- | --- |
|  | F1 (%) | IoU (%) | Prec (%) | F1 (%) | IoU (%) | Macro F1 (%) |
| Full TopoRadar-Net | 77.40 ± 1.86 | 63.16 ± 2.45 | 75.48 ± 1.15 | 50.18 ± 4.40 | 33.61 ± 3.89 | 63.79 ± 1.88 |
| w/o Local Incidence Angle (No-LIA) | 75.67 ± 1.76 | 60.89 ± 2.26 | 71.02 ± 2.80 | 52.13 ± 1.11 | 35.26 ± 1.01 | 63.90 ± 1.37 |
| w/o Cross-Attention (No-CrossAttn) | 76.85 ± 0.80 | 62.41 ± 1.06 | 70.77 ± 2.20 | 53.30 ± 0.93 | 36.34 ± 0.87 | 65.07 ± 0.53 |
| w/o Geomorphic Loss (No-GeoLoss) | 76.70 ± 0.91 | 62.22 ± 1.20 | 72.50 ± 2.15 | 51.99 ± 1.31 | 35.14 ± 1.20 | 64.35 ± 1.05 |
| w/o Aspect Alignment (No-Aspect) | 77.38 ± 2.66 | 63.18 ± 3.49 | 71.19 ± 3.40 | 53.32 ± 0.42 | 36.35 ± 0.39 | 65.35 ± 1.54 |

The ablation results in Table 5 provide nuanced empirical insights into the regional dependence of radar-topographic conditioning:

- Maritime Alpine Setting (Tromsø): In coastal Arctic mountains where fjord slopes cause severe radar foreshortening, the full TopoRadar-Net architecture records 77.40%±1.86% F1 and 63.16%±2.45% IoU. The Seed-42 paired cluster comparison with No-LIA records a +2.04% F1 difference (p<0.001), establishing sensitivity to the component rather than a causal normalization mechanism.

- Cross-Attention vs. Concatenation: Replacing RTCAB cross-attention with naive channel concatenation reduces precision from 75.48% to 70.77% in Tromsø (a −4.71% precision penalty) and incurs a paired bootstrap drop of −1.42% F1 (p=0.010). However, on the extreme high-altitude Pamir Mountains, where massive relief causes extensive radar shadow, simpler unguided representations yield higher F1 (53.30% vs 50.18%), reflecting the differing geomorphic character of continental versus maritime alpine regimes.

- Geo-Loss Trade-off and Construct Audit: In Tromsø, removing Geo-Loss changes precision from 75.48% to 72.50% and F1 to 76.70%, with a Seed-42 paired-cluster difference of +3.16% (p=0.002). However, the heuristic penalty mask contains 19,663 of 106,442 valid positive training pixels (18.47%), 824 of 86,802 validation positives (0.95%), and 492 of 56,548 held-out positives (0.87%). Across three seeds, positive-pixel recall inside the held-out penalty stratum is lower for the full model than No-GeoLoss in Tromsø (50.47%±3.18% vs. 71.29%±6.39%) and Pamir (1.08%±1.52% vs. 6.45%±6.97%). The full model’s penalty-stratum F1 is higher in Tromsø (55.10% vs. 50.34%) but lower in Pamir (0.82% vs. 2.77%). Thus Geo-Loss is a precision–recall heuristic with a documented positive-label conflict, not a physical-admissibility mechanism.

- Disclosed Regime-Dependent Sensitivity: On the pooled benchmark, the No-Aspect (65.35%) and No-CrossAttn (65.07%) ablations achieve slightly higher macro F1 than the full model (63.79%). This empirical finding is consistent with earlier observations by Gatti et al. (Gatti et al. 2026) that auxiliary terrain features do not uniformly lift accuracy across all global mountain systems. The experiment establishes regime-dependent component sensitivity but does not identify a specific layover-suppression mechanism.

Fig. 4. Systematic component-wise ablation breakdown on the Scandinavian Arctic test scene, showing performance sensitivity to Local Incidence Angle (LIA), Aspect-Look Alignment, Radar-Topographic Cross-Attention (RTCAB), and Geomorphically Bounded Loss (Geo-Loss). The x-axis is truncated to the 70%–80% F1 dynamic range.

## Topographic Radar Geometry and Slope Gradient Stratification Analysis

To evaluate the physical sensitivity and fine-grained regional behavior of Radar-Topographic Aspect-Incidence Conditioning across differing terrain morphologies, Table 6 provides a stratification analysis across two key physical axes on both held-out test scenes: (1) Radar Look Aspect Alignment (θalign=cos(α−ϕ)), partitioning terrain into radar-facing foreslopes (θalign>0.3), perpendicular cross-slopes (|θalign|≤0.3), and backslopes facing away from the radar look vector (θalign<−0.3); and (2) Terrain Slope Steepness, segmenting the landscape into runout and valley floors (<25∘), primary avalanche release chutes and track slopes (25∘–45∘), and extreme steep cliff faces (>45∘).

Table 6. Exploratory Seed-42 topographic stratification on the held-out test scenes. Performance compares TopoRadar-Net with Attention U-Net across aspect-alignment and slope zones; the multi-seed stability analysis is reported in the text.

| Region | Topographic Stratum | Area (px) | TopoRadar (Ours) |  | Attention U-Net |  | ΔF1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | F1 (%) | Prec (%) | F1 (%) | Prec (%) |  |
| Tromsø | Foreslopes (θ_align > 0.3) | 15,480 | 79.94% | 73.05% | 80.01% | 77.44% | -0.07% |
|  | Cross-slopes (¦θ_align¦ ≤ 0.3) | 3,113 | 68.93% | 63.19% | 69.43% | 73.57% | -0.50% |
|  | Backslopes (θ_align < -0.3) | 15,376 | 81.08% | 85.03% | 75.23% | 88.90% | +5.85% |
|  | Runout & Valleys (< 25°) | 12,360 | 76.76% | 70.68% | 77.91% | 76.23% | -1.15% |
|  | Avalanche Chutes (25°–45°) | 21,118 | 81.34% | 80.94% | 77.00% | 85.33% | +4.34% |
|  | Extreme Steep (> 45°) | 491 | 63.37% | 67.05% | 54.23% | 68.21% | +9.14% |
| Pamir | Foreslopes (θ_align > 0.3) | 14,743 | 51.27% | 43.87% | 50.13% | 42.84% | +1.14% |
|  | Cross-slopes (¦θ_align¦ ≤ 0.3) | 2,614 | 46.96% | 43.37% | 46.92% | 37.60% | +0.03% |
|  | Backslopes (θ_align < -0.3) | 5,222 | 53.42% | 52.36% | 48.33% | 38.41% | +5.09% |
|  | Runout & Valleys (< 25°) | 6,501 | 45.97% | 38.49% | 38.22% | 28.40% | +7.75% |
|  | Avalanche Chutes (25°–45°) | 15,875 | 55.82% | 52.33% | 56.25% | 50.55% | -0.44% |
|  | Extreme Steep (> 45°, Shadow) | 203 | 8.85% | 5.04% | 16.65% | 10.74% | -7.80% |

The Seed-42 empirical stratification provides descriptive, physics-consistent associations:

- Backslope Separation in Both Mountain Systems: Across both held-out mountain regions, TopoRadar-Net achieves its largest Seed-42 margin over Attention U-Net on slopes facing away from the radar look direction (θalign<−0.3). In Tromsø, TopoRadar-Net attains 81.08% F1 versus 75.23% for Attention U-Net, with +12.28% higher recall (rising from 65.20% to 77.48%). This pattern is consistent with the backscatter observations of Tompkin and Leinss (2021): on backslopes with high local incidence angles (55∘±20∘), undisturbed snow exhibits low backscatter and rough debris can appear strongly visible. In the high-altitude continental Pamir Mountains, the +5.09% backslope difference (53.42% vs 48.33%) is precision-associated (+13.95% precision, rising from 38.41% to 52.36%, while recall drops from 65.17% to 54.52%). These are descriptive stratum associations from Seed 42, not component-isolated causal effects.

- Separation in Active Avalanche Chutes (25∘–45∘): In the primary avalanche terrain zone (25∘–45∘ slope), TopoRadar-Net attains 81.34% F1 versus 77.00% for Attention U-Net (+4.34% F1, +5.94% IoU) in Tromsø. The foreslope difference is −0.07% in this Seed-42 descriptive comparison.

- Physical Radar Shadow Boundary: In the extreme high-altitude Pamir Mountains on slopes >45∘, performance for both networks is low (8.85% vs 16.65%, Δ=−7.80%), with debris representing only 203 pixels. The result identifies a small-sample failure stratum consistent with radar shadow; it does not prove that shadow alone causes the errors.

To test whether the exploratory Seed-42 patterns persist under training uncertainty, we repeated full-scene stratification for Seeds 42, 123, and 456 using each checkpoint’s frozen validation-selected threshold (multi-seed stratification artifact). The backslope F1 differences versus Attention U-Net average −0.34%±6.28% in Tromsø and +1.20%±7.73% in Pamir (population SD; positive in two of three seeds for each region), rather than stable >+5% effects. Tromsø foreslopes average −1.21%±0.86% (zero of three positive), while Pamir runout/valley terrain averages +5.32%±7.76% (two of three positive). Extreme-steep strata are highly seed-variable. The multi-seed check therefore narrows Table 6 to an exploratory regional characterization and provides no component-isolated directional mechanism claim.

## Extended Validation Cohort Analysis on Polar Maritime Terrain (Nuuk, Greenland)

To evaluate model stability across the broader AvalCD distribution beyond the zero-shot test scenes, Table 7 reports performance on the 11 April 2021 polar-maritime validation cohort in Southwest Greenland (used for checkpoint selection and threshold calibration; n=129 polygons, 81,595 debris pixels).

Table 7. Performance on the 11 April 2021 polar-maritime validation scene in Southwest Greenland (used for checkpoint selection and threshold calibration; n=129 polygons, 81,595 debris pixels) across three seeds (mean ± SD).

| Method | Pixel F1 (%) | IoU (%) | Precision (%) | Recall (%) |
| --- | --- | --- | --- | --- |
| SiamUNet-diff | 59.35 ± 1.58% | 42.22 ± 1.45% | 58.10 ± 0.58% | 60.79 ± 3.70% |
| SiamUNet-conc | 66.53 ± 0.97% | 49.85 ± 1.09% | 57.72 ± 2.15% | 78.66 ± 1.74% |
| ResU-Net | 67.16 ± 1.62% | 50.57 ± 1.81% | 60.10 ± 4.51% | 76.69 ± 3.48% |
| Attention U-Net | 67.01 ± 1.98% | 50.43 ± 2.22% | 57.06 ± 2.94% | 81.31 ± 0.51% |
| TopoRadar-Net (Ours) | 66.75 ± 1.96% | 50.13 ± 2.22% | 56.63 ± 3.31% | 81.50 ± 1.10% |

In this polar maritime alpine regime, characterized by open fjord expanses and rounded permafrost massifs without extreme knife-edge ridge shadows, all four modern deep change detection architectures cluster within 0.63% of each other on F1 (66.53%–67.16%). TopoRadar-Net achieves the highest detection sensitivity of all models (81.50±1.10% recall), successfully capturing the vast majority of the 129 dispersed avalanche tracks, while Attention U-Net (67.01%) and ResU-Net (67.16%) achieve marginally higher precision due to fewer false alarms on open plateau terrain. All deep architectures decisively outperform unguided differencing (59.35%, yielding a +7.40% to +7.81% F1 margin). This empirical finding corroborates the regime-dependent sensitivity documented in Section 4.4: in open sub-polar terrain with moderate relief, unguided baseline models and physics-conditioned networks perform with statistical parity, whereas rigid topographic priors provide their most decisive advantages in steep, fjord-adjacent coastal mountains (Tromsø) and high-relief continental massifs (Pamir).

# Discussion

## Empirical Associations with Radar-Topographic Geometry

The empirical performance of TopoRadar-Net relative to standard attention-augmented and Siamese convolutional baselines is consistent with known coupling between microwave backscatter and steep mountain topography. Conventional computer vision models treat SAR intensity channels as flat 2D arrays, whereas TopoRadar-Net additionally receives terrain and viewing-geometry coordinates. The reported ablations and strata quantify sensitivity to those inputs; they do not isolate a causal internal mechanism.

As demonstrated by Eq. (4), local incidence angle ψ is governed by terrain slope θ, aspect azimuth α, and satellite radar look azimuth ϕlook. On illuminated foreslopes (θalign=cos(α−ϕlook)>0), steep terrain compresses multiple ground-range cells into narrow slant-range bins. This foreshortening can elevate radar backscatter and generate bright linear ridges. RTCAB makes local-incidence and aspect-alignment features available to the SAR representation; the observed region- and stratum-specific changes are compatible with, but do not directly demonstrate, learned foreslope suppression.

Conversely, on backslopes facing away from the radar (θalign<0,ψ>50∘), undisturbed smooth snow surfaces can appear radiometrically dark due to specular reflection away from the radar antenna. Avalanche debris may then exhibit strong diffuse surface and volume backscatter (Δσ0>0). TopoRadar-Net records higher Seed-42 F1 than Attention U-Net in the reported backslope strata, but activation-level evidence would be required to attribute that difference specifically to dynamic spatial gating.

Geo-Loss encodes a heuristic slope prior rather than a physical-validity constraint. Its mask overlaps genuine positive labels, most strongly in the Nuuk 2016 training event (28.79% of valid positives), and the held-out subgroup audit records lower positive recall than No-GeoLoss in both test systems. The aggregate Tromsø precision/F1 difference and the subgroup recall penalty therefore describe a trade-off; without a matched generic regularizer they do not establish a terrain-specific mechanism.

## Cross-System Transferability Across Global Mountain Regimes

A primary challenge in machine learning for natural hazards is model overfitting to localized geographic and environmental training conditions. The AvalCD benchmark spans four distinct mountain systems characterized by starkly contrasting climatic, snowpack, and geomorphic regimes:

- Temperate Alpine Regime (European Alps): High recreation density, moderate elevation relief, and mixed maritime-continental snowpacks characterized by distinct layer metamorphism and frequent spring wet-snow transitions.

- High-Altitude Continental Regime (Pamir Mountains, Central Asia): Extreme vertical relief (>2,000 m descent paths), hyper-arid winter climate, deeply facetted depth hoar, and massive powder/slab avalanche cycles capable of burying entire valleys.

- Maritime Sub-Arctic Regime (Tromsø, Norway): Coastal mountains with heavy, wind-scoured maritime snowpacks, frequent rain-on-snow events, and polar darkness throughout mid-winter.

- Polar Fjord Alpine Regime (Nuuk, Greenland): Sub-polar maritime conditions with persistent high-velocity winds and coastal fjord relief.

The model was trained on Alps and Greenland events, selected on Alps and Greenland validation events, and evaluated without site-specific tuning on held-out Tromsø and Pamir scenes. It records 77.40%±1.86% and 50.18%±4.40% F1, respectively. These observed held-out results change climate, relief, acquisition geometry, and avalanche population simultaneously and therefore do not identify physical coordinates as the cause of transfer.

## Operational Implications for Alpine Civil Protection and Search-and-Rescue

For alpine emergency managers, highway maintenance authorities, and search-and-rescue teams, satellite-based avalanche detection is only valuable if it provides reliable, actionable intelligence during crisis cycles (Eckerstorfer et al. 2019; European Avalanche Warning Services 2022; Techel et al. 2018). Optical remote sensing is consistently thwarted during major blizzard events when rapid avalanche detection is most urgently needed. Sentinel-1 C-band SAR provides the only freely accessible, all-weather observation platform capable of continuous winter surveillance.

Large avalanches can block transportation arteries and bury infrastructure, motivating size-stratified evaluation. On the single held-out Tromsø inventory, TopoRadar-Net records mean hit rates of 84.0% for D3 and 95.8% for D4. These retrospective scene-bounded rates neither guarantee identification of future events nor measure civil-protection outcomes.

Table 8. Conceptual decision-support framework and false-alarm footprint on the evaluated finite-raster valid masks (Tromsø: 231.151 km²; Pamir: 350.338 km² on the nominal 10 m grid). The framework is illustrative and unvalidated for automated dispatch; affine areas scale proportionally with identical ha/km² rates.

| Triage Level | Score (p) | Debris Footprint | Illustrative Hazard Tier | Target Stakeholder |
| --- | --- | --- | --- | --- |
| Tier 1 | p < 0.40 | < 0.1 ha (< 10 px) | Low confidence; routine background monitoring | Warning Services (SLF, NVE) |
| Tier 2 | 0.40 ≤ p < 0.70 | 0.1–1.0 ha (10–100 px) | Moderate confidence; recommended expert review or optical cueing | Regional Forecasters |
| Tier 3 | p ≥ 0.70 | ≥ 1.0 ha (Major debris) | High confidence; prioritized inspection along transport corridors | Civil Protection / Road Auth. |
| Regional False-Alarm Footprint (Valid Area: Tromsø 231.151 km², Pamir 350.338 km²) |  |  |  |  |
| Region | Method | Detected Debris (ha) | False-Alarm Area (ha / km²) | False-Alarm Rate (ha / km²) |
| Scandinavian Arctic (Tromsø) | SiamUNet-diff | 218.33 ha | 43.13 ha (0.43 km²) | 0.19 ha / km² (Misses 121.36 ha) |
|  | Attention U-Net | 261.49 ha | 68.34 ha (0.68 km²) | 0.30 ha / km² |
|  | TopoRadar-Net (Ours) | 270.09 ha | 87.82 ha (0.88 km²) | 0.38 ha / km² (Detects 95.8% D4) |
| Pamir Mountains (Pish) | SiamUNet-conc | 163.76 ha | 302.63 ha (3.03 km²) | 0.86 ha / km² (Severe Clutter) |
|  | Attention U-Net | 150.43 ha | 242.77 ha (2.43 km²) | 0.69 ha / km² |
|  | TopoRadar-Net (Ours) | 130.25 ha | 169.89 ha (1.70 km²) | 0.48 ha / km² (30.0% Clutter Reduction) |

Table 8 summarizes the quantitative clutter footprint across mountain regimes and outlines an illustrative conceptual triage framework:

- Physical-Unit Footprints: On the Pamir valid mask (350.338 km2 nominal), TopoRadar-Net records 169.89 ha of false alarms (0.48 ha/km2), 72.88 ha (30.0%) below Attention U-Net and 132.73 ha (43.9%) below SiamUNet-conc; affine physical differences are 55.63 and 101.33 ha. In Tromsø, its density is 0.38 ha/km2 and its false-alarm area exceeds Attention U-Net, so the trade-off is region dependent.

- Calibrated Corridor Proxy: We fit Platt calibration and an F2-weighted component threshold only on the two validation scenes, then intersect ≥0.1 ha test components with a frozen OpenStreetMap major-road snapshot (30 m buffer) (OpenStreetMap contributors 2026). In Pamir (8 road-intersecting ground-truth components), TopoRadar-Net achieves 83.0%±2.3% alert precision and 62.5%±10.2% recall; Attention U-Net achieves 77.4%±6.1% and 70.8%±5.9%. Both models miss the single Tromsø corridor component in all seeds. TopoRadar Brier score improves from 0.282 to 0.208 in Tromsø and 0.462 to 0.198 in Pamir after calibration. This post-hoc proxy falsifies automated-dispatch readiness while quantifying a bounded Pamir use case.

- Cached End-to-End Profile: With the OSM snapshot cached, all measured TopoRadar-Net runs for raster loading, road-buffer rasterization, full-scene inference, calibration, connected components, and corridor intersection remain below 5 s per scene on Apple MPS. Wall-clock values vary with device warm state and are verified by bounds rather than byte identity. The profile excludes data acquisition and human review and therefore does not establish response latency or decision benefit.

## Limitations and Future Research

Several physical and technical limitations warrant disclosure:

- Augmentation Convention: The released checkpoints use the disclosed checkpoint-era convention in which aspect sine/cosine components are sign-adjusted under reflections while the precomputed orbital look-alignment channel is spatially transformed without directional recomputation; orthogonal rotations similarly preserve channel values. The resulting training tensor is not fully equivariant under these transformations. Consequently, the directional results are interpreted as empirical sensitivity rather than a causal validation of the manifold representation. A future confirmatory rerun should compare a fully equivariant augmentation against the released convention.

- Heuristic Slope Prior: Geo-Loss penalizes positive predictions whenever slope is below 5∘ or above 65∘, including genuine mapped debris. The conflict is concentrated in one training event but is present in every split; held-out subgroup recall is lower than No-GeoLoss. Future work should compare label-aware and generic regularizers on a fresh protected cohort.

- DEM Quality and Geometric Resolution: TopoRadar-Net relies on the Copernicus GLO-30 DEM. While GLO-30 provides global coverage, localized vertical errors or smoothing over sharp knife-edge arêtes can propagate inaccuracies into the local incidence angle calculation. In regions with available high-resolution airborne LiDAR or stereo-photogrammetric DEMs (1–5 m), topographic conditioning can be further refined.

- Dielectric Attenuation in Wet Snow Cycles: Liquid water changes microwave permittivity and can reduce C-band contrast during rain-on-snow events or warm spells. Bitemporal differencing may therefore mask debris or create broad absorption anomalies. Future work should integrate multi-temporal radiometry or polarimetric decomposition parameters.

- Temporal Revisit Constraints: A several-day repeat interval can aggregate multiple releases into one composite deposit. Sentinel-1C and Sentinel-1D launched on 5 December 2024 and 4 November 2025, respectively (European Space Agency 2026); the expanded constellation improves continuity but does not remove acquisition-latency or event-timing ambiguity.

- Operational Proxy, Not Stakeholder Validation: The calibrated corridor analysis uses a committed OpenStreetMap snapshot and retrospective labels. It measures component calibration, route proximity, and cached processing time, not dispatch quality, road closure decisions, or stakeholder benefit. The Tromsø miss and the small number of corridor-intersecting avalanches preclude deployment claims.

# Conclusion

Automated mapping of snow avalanche debris in steep alpine terrain is a vital capability for mountain natural hazard risk reduction and emergency response. In this study, we addressed severe topographic-radar distortion in spaceborne Sentinel-1 C-band SAR change detection with TopoRadar-Net, which combines multi-scale Radar-Topographic Cross-Attention Blocks (RTCAB) and a heuristic geomorphic slope-prior loss (Geo-Loss).

Our key scientific and methodological findings include:

- Theoretical Formulation of 2D Convolutional Inadequacy in Mountain SAR: We formulated (Proposition 2) the geometric limitations of shift-invariant 2D spatial linear convolutions across sloped mountain facets, showing that projecting terrain into a continuous 4D aspect-look manifold (M) provides continuous coordinates to condition feature representations without angular branch-cuts.

- Physics-Guided Cross-Attention Sensitivity: TopoRadar-Net conditions SAR change representations on directional aspect-look alignment (θalign) and local incidence angle (ψ). Component-removal and stratum analyses quantify sensitivity to these inputs, but do not isolate a causal internal suppression or amplification mechanism.

- Geomorphic Prior Trade-off: Removing the heuristic slope prior produces a −3.16% Seed-42 paired-cluster F1 difference in Tromsø (p=0.002), but raises three-seed positive recall inside the penalty mask from 50.47% to 71.29% in Tromsø and from 1.08% to 6.45% in Pamir. Because 18.47% of training positives fall inside the mask, the term is not interpreted as a physical-admissibility mechanism.

- Cross-System Generalization and Stratification Findings: Across the AvalCD benchmark spanning four global mountain systems, TopoRadar-Net achieves 63.79%±1.88% pooled macro F1, compared with Attention U-Net (63.41%), SiamUNet-conc (63.13%), ResU-Net (62.92%), and SiamUNet-diff (54.87%). The three-seed gain over SiamUNet-diff is significant (+8.92%, p=0.021), while comparisons with competitive baselines indicate parity (p=0.869 vs Attention U-Net; p=0.459 vs ResU-Net). Seed-42 backslope differences do not remain stable across seeds: the mean differences are −0.34%±6.28% in Tromsø and +1.20%±7.73% in Pamir.

- Operational Proxy and Failure Boundary: Validation-calibrated road-corridor alerts achieve 83.0% precision and 62.5% recall in Pamir, with all cached TopoRadar scene runs below 5 s, but miss the single Tromsø corridor avalanche in all seeds. This retrospective proxy quantifies practical behavior while explicitly rejecting automated-dispatch readiness.

Overall, terrain conditioning is useful but not uniformly superior. The main contribution is a reproducible account of where it helps, where it fails, and what evidence is still required before operational use.

# Declarations

Ethical Approval and Consent: Not applicable. Consent for Publication: Not applicable. Funding: No external grant funding was received. Competing Interests: The authors declare no competing financial or non-financial interests.

Availability of Data and Materials: The open-access AvalCD benchmark is deposited on Zenodo. Sentinel-1 SAR and Copernicus DEMs are provided under Copernicus Open Access. Persistent links are withheld for double-blind review.

Code Availability: All model architectures, training protocols, and verification suites supporting this study are available in an open-access repository; the URL is withheld for double-blind review.

Use of Artificial Intelligence: AI-assisted technologies were used solely for language editing and code formatting. All scientific conceptualizations, formulations, and results were executed and approved by the authors.

Authors’ Contributions: Author details are withheld for double-blind review and will be provided in camera-ready. All authors approved the final manuscript.

Bandara, Wele Gedara Chaminda, and Vishal M. Patel. 2022. “A Transformer-Based Siamese Network for Change Detection.” IEEE International Geoscience and Remote Sensing Symposium (IGARSS), 207–10. https://doi.org/10.1109/IGARSS46834.2022.9883686.

Bianchi, Filippo Maria, Jakob Grahn, Markus Eckerstorfer, Eirik Malnes, and Hannah Vickers. 2021. “Snow Avalanche Segmentation in SAR Images with Fully Convolutional Neural Networks.” IEEE Journal of Selected Topics in Applied Earth Observations and Remote Sensing 14: 75–82. https://doi.org/10.1109/JSTARS.2020.3036914.

Bühler, Yves, Elisabeth D. Hafner, Benjamin Zweifel, Mathias Zesiger, and Holger Heisig. 2019. “Where Are the Avalanches? Rapid SPOT6 Satellite Data Acquisition to Map an Extreme Avalanche Period over the Swiss Alps.” The Cryosphere 13 (12): 3225–38. https://doi.org/10.5194/tc-13-3225-2019.

Cao, Chen, Ji Bai, Tianhao Song, et al. 2026. “Integrating Information Value Method and Extreme Gradient Boosting for Debris Flow Susceptibility Assessment on the Piedmont Clinoplain Considering Watershed Erosion.” Journal of Mountain Science 23 (4): 1614–34. https://doi.org/10.1007/s11629-025-9805-y.

Caye Daudt, Rodrigo, Bertrand Le Saux, and Alexandre Boulch. 2018. “Fully Convolutional Siamese Networks for Change Detection.” 2018 25th IEEE International Conference on Image Processing (ICIP), 4063–67. https://doi.org/10.1109/ICIP.2018.8451652.

Chen, Hao, Zipeng Qi, and Zhenwei Shi. 2022. “Remote Sensing Image Change Detection with Transformers.” IEEE Transactions on Geoscience and Remote Sensing 60: 1–14. https://doi.org/10.1109/TGRS.2021.3095166.

Eckerstorfer, Markus, Eirik Malnes, and Karsten Müller. 2017. “A Complete Snow Avalanche Activity Record from a Norwegian Forecasting Region Using Sentinel-1 Satellite-Radar Data.” Cold Regions Science and Technology 144: 39–51. https://doi.org/10.1016/j.coldregions.2017.08.004.

Eckerstorfer, Markus, Karsten Müller, Eirik Malnes, and Hilde Daugstad Oterhals. 2021. “Manual and Automatic Detection of Dry Snow Avalanches in Sentinel-1 SAR Images.” IEEE International Geoscience and Remote Sensing Symposium (IGARSS), 247–50. https://doi.org/10.1109/IGARSS47720.2021.9554370.

Eckerstorfer, Markus, Hannah Vickers, Eirik Malnes, and Jakob Grahn. 2019. “Near-Real Time Automatic Snow Avalanche Activity Monitoring System Using Sentinel-1 SAR Data in Norway.” Remote Sensing 11 (23): 2863. https://doi.org/10.3390/rs11232863.

European Avalanche Warning Services. 2022. Avalanche Size Classification Standards (Classes D1 Through D5). Https://www.avalanches.org/standards/avalanche-size/.

European Space Agency. 2026. Sentinel-1: Radar Vision for Copernicus. Https://www.esa.int/Applications/Observing_the_Earth/Copernicus/Sentinel-1.

Garova, Elena, Bogdan Chadromtsev, Alexander Pedanov, et al. 2025. “A General Methodological Framework for Hazard Assessment in Remote Mountain Areas Combining Geomorphological Mapping with UAV Survey.” Journal of Mountain Science 22 (3): 763–75. https://doi.org/10.1007/s11629-024-9096-8.

Gatti, Mattia. 2026. “AvalCD.” Zenodo. https://doi.org/10.5281/zenodo.15863589.

Gatti, Mattia, Alberto Mariani, Ignazio Gallo, and Fabiano Monti. 2026. “Large-Scale Avalanche Mapping from SAR Images with Deep Learning-Based Change Detection.” arXiv Preprint arXiv:2603.22658. https://arxiv.org/abs/2603.22658.

Gelato, Riccardo, Carlo Sgaravatti, Jakob Grahn, Giacomo Boracchi, and Filippo Maria Bianchi. 2026. “Promptable Foundation Models for SAR Remote Sensing: Adapting the Segment Anything Model for Snow Avalanche Segmentation.” Remote Sensing 18 (3): 519. https://doi.org/10.3390/rs18030519.

Guo, Fei, Zhi-zhen Xu, Jilei Hu, Jie Dou, Xiaowei Li, and Qinglin Yi. 2025. “Forecasting Step-Like Landslide Displacement Through Diverse Monitoring Frequencies.” Journal of Mountain Science 22 (1): 122–41. https://doi.org/10.1007/s11629-024-9064-3.

Guo, Junqi, Wenfei Xi, Zhiquan Yang, et al. 2024. “Landslide Hazard Susceptibility Evaluation Based on SBAS-InSAR Technology and SSA-BP Neural Network Algorithm: A Case Study of Baihetan Reservoir Area.” Journal of Mountain Science 21 (3): 952–72. https://doi.org/10.1007/s11629-023-8083-9.

Liu, Yang, Xi Chen, Yubao Qiu, Jiansheng Hao, Jinming Yang, and Lanhai Li. 2021. “Mapping Snow Avalanche Debris by Object-Based Classification in Mountainous Regions from Sentinel-1 Images and Causative Indices.” CATENA 206: 105559. https://doi.org/10.1016/j.catena.2021.105559.

OpenStreetMap contributors. 2026. OpenStreetMap. Https://www.openstreetmap.org/copyright.

Sajjad, Meer Muhammad, Juanle Wang, Daqing Ge, Rehan Khan, Izhar Ahmed, and Khan Zada. 2024. “Assessing the Potential Landslide Risk Identification in the Northern Section of CPEC Route Pakistan Based on Multi-Temporal InSAR Approaches.” Journal of Mountain Science 21 (12): 4131–48. https://doi.org/10.1007/s11629-024-8871-x.

Small, David. 2011. “Flattening Gamma: Radiometric Terrain Correction for SAR Imagery.” IEEE Transactions on Geoscience and Remote Sensing 49 (8): 3081–93. https://doi.org/10.1109/TGRS.2011.2120616.

Techel, Frank, Christoph Mitterer, Elisabetta Ceaglio, et al. 2018. “Spatial Consistency and Bias in Avalanche Forecasts—a Case Study in the European Alps.” Natural Hazards and Earth System Sciences 18 (10): 2697–716. https://doi.org/10.5194/nhess-18-2697-2018.

Tompkin, Cedric, and Silvan Leinss. 2021. “Backscatter Characteristics of Snow Avalanches for Mapping with Local Resolution Weighting.” IEEE Journal of Selected Topics in Applied Earth Observations and Remote Sensing 14: 4452–64. https://doi.org/10.1109/JSTARS.2021.3074418.

Veitinger, J., B. Sovilla, and R. S. Purves. 2014. “Influence of Snow Depth Distribution on Surface Roughness in Alpine Terrain: A Multi-Scale Approach.” The Cryosphere 8 (2): 547–69. https://doi.org/10.5194/tc-8-547-2014.

Vickers, Hannah, Markus Eckerstorfer, Eirik Malnes, Yngvar Larsen, and Heidi Hindberg. 2016. “A Method for Automated Snow Avalanche Debris Detection Through Use of Synthetic Aperture Radar (SAR) Imaging.” Earth and Space Science 3 (11): 446–62. https://doi.org/10.1002/2016EA000168.

Wang, Di, Rong-hao Yang, Xiao Wang, et al. 2023. “Evaluation of Deep Learning Algorithms for Landslide Susceptibility Mapping in an Alpine-Gorge Area: A Case Study in Jiuzhaigou County.” Journal of Mountain Science 20 (2): 484–500. https://doi.org/10.1007/s11629-022-7326-5.

Zhang, Zhengxin, Qingjie Liu, and Yunhong Wang. 2018. “Road Extraction by Deep Residual U-Net.” IEEE Geoscience and Remote Sensing Letters 15 (5): 749–53. https://doi.org/10.1109/LGRS.2018.2802944.

Zhou, Zhen-kai, Xin Yao, Ren-jiang Li, et al. 2023. “Deformation Characteristics and Mechanism of an Impoundment-Induced Toppling Landslide in Baihetan Reservoir Based on Multi-Source Remote Sensing.” Journal of Mountain Science 20 (12): 3614–30. https://doi.org/10.1007/s11629-023-7903-2.